| Kenwood TH-F6A 144/220/440 Triband Transceiver |  |
| --- | --- |
| Storing a frequency in memory |  |
| 1. Select VFO Mode | Press [VFO] Press [A/B] to select the A Band |
| 2. Enter Frequency | Press [ENT]. Direct enter the frequency |
| 3. Select Offset | Press [F] [REV] toggles through +,-,none |
| 4. Activate Tone | Press [TONE]. Verify T is displayed |
| 5. Select Tone Freq | With Tone=ON, Press [F] [TONE] Turn Tuning control or []/[] controls to select Press [/OK] or [MNU] to accept |
| 6. Store in Mem Channel | Press [F]. Mem Channel Number blinks Turn Tuning control or []/[] controls to select Press [/OK] or [MR] to save |
| 7. Select Memory Mode | Press [MR] |
| Other Operations |  |
| Clear Mem Channel | Select the memory channel to clear Turn Power Off Press [MR] + (Power) Press [MR] or [/OK] or [MNU] to clear To Cancel Clear, press any other key |
| Copy Mem to VFO | Press [F], [VFO] |
| Set Power | Press [LOW] cycles thru H, L, EL |
| Lock the Keyboard | Press [F] and hold until the Lock symbol appears. Repeat to unlock |

| Other Operations |  |  |
| --- | --- | --- |
| Single Band Operation | Press [A/B] to select the band you want to display Press [F], [A/B] to cause the display to show that band only. Press [F], [A/B] again to return to dual band operation. |  |
| Weather | Press [INFO] To exit, Press [VFO] or [MR] |  |
| Scan, Memory | To start, press [MR] (1 s) To stop, press [MR] or [PTT] |  |
| Scan Mem lockout | Press [MR]. Select the Channel to lock out Press [F], [MHz]. An * appears above the ch # Press [F], [MHz] again to unlock |  |

| Set CTCSS | When On, hears calls when selected CTCSS tone is received. Press [TONE]. Verify CT is displayed |
| --- | --- |
| Select CTCSS Tone | With CTCSS=ON, Press [F] [TONE] Turn Tuning control or []/[] controls to select Press [/OK] or [MNU] to accept |

| Select Commercial Stations | Press [A/B] to select B Band Commercial AM: Press [ENT], then 0 [MHz] 9 1 0 Commercial AM: Press [ENT], then 1 [MHz] 2 2 0 Commercial FM: Press [ENT], then 1 0 3 [MHz] 7 0 0 |
| --- | --- |
| 26-Aug-2005 |  |