| Radio Shack HTX-202 2 Meter FM Transceiver |  |
| --- | --- |
| Storing a frequency in memory |  |
| 1. Select VFO Mode | Press [VF] |
| 2. Enter Frequency | Enter the last 4 digits of the frequency. For instance, for 147.570, enter 7570. On a mistake, re-enter the digits. |
| 3. Select Tone Freq | Press [F] [M-SET] Press SC or SC to step up or down through the menus. At tc, rotate the tuning knob to select the Transmit Subaudible Tone. Press PTT to exit the menu and save the configuration. |
| 4. Turn on Tone Freq | Press [F] [T-SQL]. Confirm the T-SQL symbol is on |
| 5. Set the Offset | Press [F] [+/-] to toggle between +, -, or no offset. |
| 6. Store in Mem Channel | Press [F] and rotate the tuning knob to select the memory channel for writing. Press and hold [F] [M-WR] for 1 second to store the frequency into the selected channel. |
| 7. Select Memory Mode | Press [MR] |
| Other Operations |  |
| Clear Mem Channel | Press [MR] and select the memory channel to clear. Press [F] [M-CLR] to clear |
| Copy Mem to VFO | Press [MR] Select the channel, then press [F] [M-CLR] Press [VF] |

| Other Operations |  |  |
| --- | --- | --- |
| Set Power | Press the Low Power button on top of the radio; IN for Low power, OUT for High Power. |  |
| Lock the Keyboard | Press [F] [LOCK]. The Lock symbol appears. Repeat to unlock. |  |
| Power Save | Press [F] [SAVE]. The Save symbol appears. Repeat to unlock. |  |
| Memory Scan | Press [MR] Press and hold SC or SC to scan in that direction. |  |
| Frequency Scan | Press [VF] Press [F] [M-SET] Press SC or SC to set the Scan limits. Press PTT to exit the menu and save the configuration. Press and hold SC or SC to scan in that direction. |  |
| Configuration Menu | Code Default Explanation oS 0.600 Duplex separation tc 88.5 Xmit Subaudible rc 88.5 Recv subaudible Sr 20 Frequency Step Sc ti Scan resume Sd 2.0 San Delay Time | S1 144.00 Low scan range S2 148.00 High scan range Ud dn Vacant scan dir PS 1-16 Pwr Save DutyC tE oFF Transmit inhibit to oFF Transmit timeout Lb 4 Freq Lock time Ar oFF Touchtone reply |
| 25-Aug-2005 |  |  |