| Yaesu FT-50R 144/440 Dualband Transceiver |  |
| --- | --- |
| Storing a frequency in memory |  |
| 1. Select VFO Mode | Press [V/M], less than 1 second. The Channel number should be out. |
| 2. Enter Frequency | Directly enter the frequency, 6 digits |
| 3. Select Offset | FT-60R will automatically assign the offset based on the band plan |
| 4. Activate Tone (pg 22) | - Press [F/W] then [1(SQ TYP)]. - Use Tuning control to select “Tone”, PTT to save. - Press [F/W] then [2(CODE)]. - Use Tuning control to select Tone Freq, [F/W] to save. |
| 6. Store in Mem Channel | - Press [F/W] for 1 second. Mem Channel Number blinks showing the next unassigned channel number. - Use Tuning control to select Mem channel. - Press [F/W] to save (listen for 1 beep), then [PTT]. |
| 7. Delete a Mem Channel | - Press [F/W] to enter MR mode. - Use Tuning control to select Mem channel to delete. - Briefly press [HM/RV]; do not hold. - Display reverts to Channel 1. |
| 7. Select Memory Mode | Press [V/M]. “MEMORY” is briefly displayed, then frequency display with channel number. |

---------------------------------------------------------------------------------------------------------

| Other Operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Set Power Level (pg 15) |  | - Press[F/W], then [3 (TX PO)] - Use Tuning control to select LOW, MED, HIGH - Press [PTT] to save. |  |  |  |
| Battery Saver |  | - Press [F/W], then [0 (o)SET]. - Use Tuning control to select 41: RXSAVE. - Press [F/W] to select sleep time or OFF - Use Tuning control to select 43: TXSAVE. - Press [F/W] to select ON or OFF - To exit, press [PTT] to save. |  |  |  |
| Lock Keyboard |  | - Press [F/W], then [6 LOCK] to toggle |  |  |  |
| Scanning |  | - Press ▲MHz or ▼MHz |  |  |  |
| Set Menu Mode |  | 1. Press [F/W], then use Tuning control to select 1 .. 55 2. Press [F/W] again for submenus. 3. Use Tuning control to select 43: TXSAVE. 4. To exit, press [PTT] to save. |  |  |  |
| 1.AutoPowerOff 2.Beep Option 3.ARTS 4.ARS 5.BCLO 6.Beep 7.Bell 8.Busy LED 9.CPUClockFreq 10.CWID 11.StoreCWID | 12.DCVolts 13.DCS Code 14.DCS N/R 15.DTMF delay 16.DTMF speed 17.DTMF Writer 18.EAI 19.EdgeBandBeep 20.EMG S 21.Internet Link 22.WIRES DTMF |  | 23.Wires MemReg 24.LampKeybd 25.Lock Combinatn 26.MONI Sw.F 27.DisplayName 28.StoreChName 29.CTSS Paging 30.PageAutoAns 31.PageCode 32.PageCodeCTCSS 33.EnablePassWd | 34.Store PassWd 35.ScanResumeMode 36.REV/HM 37.RF Squelch Adj 38.RepeaterShiftDir 39.Priority Revert 40.ReceivingMode 41.RxBatterySaver 42.SmartSearchMode 43.MemScanMode 44.EnableScanLamp | 45.RepeaterShiftOffset 46.MemScanSkipMode 47.CTSCC/DCS Split 48.SelectToneEncoder 49.SynthesizerSteps 50.CTCSS ToneFreq 51.TOT Time 52.Enable TX LED 53.TxBatterySaver 54.VFO Band Edge 55.Wide/NarrowDev |