Microphone (MIC) Switches

| DWN | lower VFO or MR; hold 1 sec to start scanning |
| --- | --- |
| UP | higher VFO or MR; hold 1 sec to start scanning |
| PTT | push-to-talk button |
| ACC | activate Priority Channel monitoring system |
| VFO/MR | switch between VFO and MR modes |
| LOCK | disables the MIC buttons |
| LAMP | up to enable lamp for night operation |
| P1 | select tone squelch type: CTSSSW, DCS, BELL, or OFF (no tone), p. 28 |
| P2 | power output level; LOW (5w), MID2 (10w), MID1(20w) and HIGH (35w UHF/50w VHF) |

Front Panel Switches

| PWR | power on/off |
| --- | --- |
| DISP SS | change display |
| MAIN DIAL | move up/down between frequencies or Memory Channels, within the selected band of operation. See page 20 for setting the 1 MHz or 10 MHz ranges. |
| VOL | change volume |
| SQL | adjust squelch – to set, turn clockwise slightly past the point where the background band noise is muted, turn all the way counterclockwise for maximum sensitivity to weak signals |
| ◄ RV | Reverse |
| SET - VU | switch between VHF/UHF in VFO mode, Frequency/Alpha in Memory Mode |
| ► RP (RPTR) | Repeater Shifts are ARS (automatic repeater shift), - (minus), + (plus), OFF (simplex), etc |

Display Options

| Voltage Display | DC supply voltage |
| --- | --- |
| Control Mode | V is VFO, M is Memory, m is Memory Only, P is Priority Channel Scanning, p is Programmable Memory Scan (Band Limits) |

Locking

| MIC keypad | use the LOCK switch on the right side of the microphone |
| --- | --- |
| Front panel | menu item “20LOCK” |
| RF Squelch | press menu item “27RfSql”; select the squelch threshold (S-3, S-5, S-FULL, OFF). Save the new setting. Now, only signals exceeding the prescribed S-meter level will open the squelch. Now set SQL as above. |

###### Memory Operations

Memory Storage

- In VFO mode, select the desired frequency, repeater shift, CTCSS tone, TX power level.

- Press and hold in the VFO/MR button on the MIC for 1 second.

- Within 5 seconds, use the MAIN DIAL (or MIC UP/DOWN buttons) to select the desired memory channel.

- Press the VFO/MR button again momentarily to store the data into the memory channel.

Naming and Displaying Memories

- Recall the memory to be named.

- Press and hold SET for 1 second, then rotate MAIN DIAL to menu item “02ALPHc”.

- For each blinking position, use ◄ or ► to choose the desired character.  Press SET to set the character and move one position to the right.

Loading a Memory into the VFO – or how to start from an existing Memory without Erasing It

- Recall the Memory Channel.

- Press and hold the MAIN DIAL for one second.  The radio will switch to VFO mode, with all the memory channel settings as they were.

- Continue with the Memory Storage process or VFO tuning.

Memory Recall

- If in VFO mode, momentarily press VFO/MR button.

- Use the MAIN DIAL knob or the MIC UP/DOWN buttons to select the memory channel. Or, use direct keypad memory recall: press the digits on the keypad, followed by the * key.

Menu Setting

- Press and hold SET key for 1 second.

- Turn the MAIN DIAL to the menu item.

- Press ►to change between settings.  Note that the display may show a ▼ character to show what to press to change settings.

- Press and hold SET for 1.5 sec to save the new setting.

| Menu # | Name | Settings | Explanation | Page |
| --- | --- | --- | --- | --- |
| 01 | ALPH | On, Off | Enable/disable the Alphanumeric Display | 53 |
| 02 | ALPHc | -- | Allows alphabetic entry of names for memory channels | 36 |
| 03 | APO | 1 – 12 hrs, OFF | Automatic Power Off. Turns the radio completely off after the defined # of hours with no PTT activity. | 46 |
| 04 | not used |  |  |  |
| 05 | ARTS | RX, TX, TRX, OFF | Sets the Auto Range Transpond System | 43 |
| 06 | BEEP | On, Off | Turns on/off the audible feedback for the buttons; note that the volume level of the beeps is fixed | 22 |
| 07 | CWID | On, Off | Turns on the CD ID (Morse Identifier) | 44 |
| 08 | CWID# | -- | Allows setup of CW ID (Morse Identifier); part of the ARTS feature | 44 |
| 09 | D Con | 0 – 12 | Set the display contrast; default is 6 | 23 |
| 10 | D Dim | d1, d2, d3, d4 or OFF | Set the display illumination | 23 |
| 11 | DCIN | -- | Indication of the supply voltage | 54 |
| 12 | DCS# | 104 std DCS codes | Set the DCS code to one of the 104 standard codes; default is 023 | 54 |
| 13 | DCS | -- | Active the DCS Code Search Scanner | 30 |
| 14 | DCSnr | TRX NOR, RX REV, TX REV, TRX REV | Normally, this should not be changed. See manual. Selection of Normal or Inverted DCS code | 48-49 |
| 15 | DTMFd | 50, 250, 450, 750, 1000 ms | Set delay between the time the SET key is pressed and the first DTMF digit is sent | 32 |
| 16 | DTMFs | 50 ms, 100 ms | Set speed at which DTMF digits are sent | 32 |
| 17 | DTMFw | -- | Sets/Accesses the autodialer feature | 31 |
| 18 | FAN | AUTO, TX, AUTO/TX, OFF | Changes the operating mode of the cooling fan. AUTO = high speed fan when temp rises, low speed otherwise; TX = high speed during Tx + 30 sec, otherwise low; AUTO/TX = AUTO and TX conditions; OFF = the fan is continuously low speed | 47 |
| 19 | LckTx | BAND A, BAND B, BOTH, OFF | Lock out PTT and prevent transmission BAND A = VHF, BAND B=UHF | 24 |
| 20 | LOCK | On, Off | Lock front panel | 20 |
| 21 | PCKT | 1200 bps, 9600 bps | Sets the packet baud rate; can be set separately for each band | 17,45 |
| 22 | Pg: ◄ | TONE, PRI, SCAN UP, SCAN DN, RPTR, REVERSE, T.BURST, TX PWR, HOME CH, VFO/MR | Programming the ◄ key assignment, default is REVERSE | 47 |
| 23 | Pg: ► |  | Programming the ► key assignment, default is RPTR | 47 |
| 24 | Pg:P1 |  | Programming the P1 key assignment, default is TONE | 47 |
| 25 | Pg: P2 |  | Programming the P2 key assignment, default is TX PWR | 47 |
| 26 | Pg: AC |  | Programming the ACC key assignment, default is PRI | 47 |
| 27 | RfSql | S-3, S-5, S-FULL, Off | Set RF squelch so that only signals exceeding the prescribed S-meter level will open the squelch | 18 |
| 28 | RPTR | ARS, Shift -, Shift +, OFF | Setting the Repeater Shift Direction | 57 |
| 29 | SCAN | Busy, Time | Sets the scan-resume mode: Busy stops as long as signal is present, Time stops for 5 seconds only | 38 |
| 30 | Shift | Multiples of 50kHz | Sets the default repeater shift; factory set at 600 kHz VHF and 5.0 MHz UHF (US) | 25 |
| 31 | SKIP | SKIP/STOP | Marks memory channels to be skipped during scanning, see manual | 39 |
| 32 | STEP | 5.0, 10.0, 12.5, 15.0, 20.0, 25.0, 50.0 | Sets KHz step size for the current operating band; default is 5.0 for VHF, and 25 for UHF. | 22 |
| 33 | TONE | ENC, ENC/DEC, BELL, DCS, OFF | Select the CTCSS/DCS Operation Mode | 28,58 |
| 34 | TONEf | 39 std CTCSS Tones | Select the CTCSS Tone Frequency; default is 100.0 | 28,58 |
| 35 | TOT | 1 – 60 min., Off | Sets the Time Out Timer, which is the maximum TX time for the MIC. Default is 6 minutes. | 46 |
| 36 | TxNar | On, Off | Reduce MIC gain; set the transmitter deviation to Narrow, to minimize interference on tightly clustered bands | 32 |
| 37 | TxPwr | HIGH, MID1, MID2, Low | set the TX output power; used to set for memory channel storage | 24,34 |
| 38 | AM | INHIBIT, AM, AUTO | Set AM Model Activation: inhibit = don’t allow activation of AM mode within 110-136 MHz, AM = force the mode to be AM within the range, auto = automatic mode selection within the range | 21 |