### VX-5R Operation Notes

Controls and Connections

| Control | Description | Page |
| --- | --- | --- |
| PTT | push-to-talk button | 15 |
| MONI | press to disable squelch, allowing you to hear very weak signals near the background noise level | 2, 29 |
| VOLUME | knob to adjust volume | 12 |
| TX/BUSY | lamp glows red during Transmission, green during Receive | 2 |
| DIAL | main tuning dial | 14 |
| MIC/SP jack | 4-conductor miniature jack provides connection points for external MIC, audio, earphone, PTT and ground | 2 |
| EXT DC | coaxial DC jack for charging battery or powered operation | 2 |
| PWR | press and hold this switch for one second to turn power on/off | 12 |
| L (black button) | lamp for display and keypad for night operation | 18, 2 |
| F/W | used to select alternate function for keypad buttons | 2 |

Front Panel Display

| Item | Description | Page |
| --- | --- | --- |
| Va or Vb | indicates the operating band in VFO mode | 3 |
| Freq | Current Frequency/Data field | 20, 3 |
| F, L, S or W | F = alternate key functions active L = lock feature active S = set mode active W = memory write feature active | 34 60 42 |
| ►►► | S- and TX Power Meter | 56, 3 |
| NFM, AM or W-FM | Operating Mode | 15, 23, 3 |

Display Icons (from left to right on bottom line of display)

| Icon | Description | Page |
| --- | --- | --- |
| DW | Dual Watch Active | 50 |
| + or - | Repeater Shift Direction | 24 |
| T | CTCSS Encoder Active | 26 |
| SQ | CTCSS Decoder Active | 26 |
| DCS | DCS (Digital Coded Squelch) Active | 27 |
| Clock | Automatic Power-Off Active | 33 |
| L ) ) ) | Low TX Power Selected | 30 |
| Bell | Bell Alarm Active | 29 |
| Telephone | DTMF Autodialer Active | 36 |
| Battery (S) | Battery Saver Active | 32 |
| Broken Battery | Low Battery Indicator | 10 |

Memory Recall

- If operating in VFO mode, press the MR key.

- Rotate the DIAL to the desired channel.  OR, key in the memory channel number and press the MR key.

Memory Storage

- In VFO mode, select the desired frequency.  Set up any CTCSS or DCS tones, repeater offset, and power level as needed.

- Press and hold F/W for 1 second.

- Within 5 seconds of releasing F/W, rotate the DIAL to select the desired memory channel.  Note that channels marked with a * are free.

- Press F/W to store the frequency into memory.

- Note that you are still operating in VFO mode.

Changing Menu Settings

- Press F/W, and then press SET.

- Turn the DIAL to the menu item.

- Press SET momentarily, and then rotate the DIAL to adjust or select the setting.

- Press PTT momentarily to save the new setting.

Keypad Functions

| Key | Press |  | Press F/W and then press |  | Press and Hold |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band (Set AR) | Band = set frequency band | 13 | Set = enter Set (menu) Mode |  | AR = activate ARTS | 13, 30 |
| HM/RV (EMG) | RV = reverse Transmit/Receive frequencies | 25 | HM = switch to Home frequency | 43 | EMG = activate the Emergency feature | 37 |
| TX PO (Lock) | TX PO = select desired transmit output level (Full, L1, L2, L3) | 30, 15 | LOCK = Key Lockout | 34 | [none] |  |
| MR (SKP SC) | MR = switch to Memory Mode | 43 | SKP = designate a channel to be skipped during scanning | 48 | SC = initiate Memory Channel scanning | 47 |
| VFO (DW SC) | VFO = switch to VFO mode | 43 | DW = Dual Watch | 50 | SC = initiate VFO scanning | 47, 14 |
| 1 (TN FRQ) | digit 1 |  | TN = CTCSS or DCS operation | 26 | FRQ = enable display of sub-band frequency | 20 |
| 2 (CD TAG) | digit 2 |  | CD = select CTCSS tone or DCS code number | 26 | TAG = select display type for memory operation | 44 |
| 3 (DT VLT) | digit 3 |  | DT = DTMF autodialer function | 36 | VLT = display battery type and voltage | 35 |
| 4 (MG DSP) | digit 4 |  | MG = activate the Memory Group mode | 45 | DSP = switch display between large and small character mode | 20 |
| 5 (AP ICO) | digit 5 |  | AP = Automatic Power Off | 33 | ICO = switch display between alpha-numeric and icon formats | 56 |
| 6 (RP ONT) | digit 6 |  | RP = select direction of uplink freq shift | 25 | ONT = display operating and total transmit time for this session | 33 |
| 7 (ST TMP) | digit 7 |  | ST = select synthesizer steps for VFO operation | 22 | TMP = display current temperature of inside case | 35 |
| 8 (BRO) | digit 8 |  | [none] |  | BRO = display current barometric pressure | 55 |
| 9 (BP ALT) | digit 9 |  | BP = turn keypad beeper on/off | 35 | ALT = display current altitude | 55 |
| 0 (SQ WX) | digit 0 |  | SQ = adjust squelch threshold level | 12 | WX = recall the “Weather” broadcast channel | 17 |

Note: the numbers refer to the manual page numbers

| Menu # | Name | Settings | Explanation | Page |
| --- | --- | --- | --- | --- |
| 01 | SQUELCH | 0 – 15 | Set AM and FM-N squelch threshold level; default is 01 | 12 |
| 02 | WFM SQL | 0 – 8 | Set the FM-Wide squelch threshold level; default is 02 | 61 |
| 03 | NAME SET | -- | Store alphanumeric tags | 44, 61 |
| 04 | ICON | -- | Icon select | 56 |
| 05 | VFO | ALL, BAND | select/disable the VFO band edge | 22 |
| 06 | ARS | On./Off | enable/disable the Automatic Repeater Shift | 24 |
| 07 | RPT SHIFT | -RPT, +RPT, SIMP | set the direction of the repeater shift | 25 |
| 08 | SHIFT | 0 – 99.95 MHz | set the magnitude of the repeater shift | 25 |
| 09 | VFO SPLIT | On/Off | enable/disable VFO Split operation | 21 |
| 10 | VFO LINK | On/Off | enable/disable VFO Link feature | 22 |
| 11 | VFO STEP | 5/9/10/12.5/15/ 20/25/50/100 KHz | set the size of the VFO steps | 22 |
| 12 | RESUME | 5 SEC, BUSY, HOLD | select the scan resume mode | 46, 63 |
| 13 | SCAN LAMP | On/Off | when on, activate lamp on scan stop | 50 |
| 14 | RX SAVE | OFF, 200 mS, 300 mS, 500 mS, 1S, 2S | select the RX mode Battery Saver Interval | 32 |
| 15 | TX SAVE | On/Off | enable/disable the Transmit battery saver feature | 32 |
| 16 | APO | OFF, 30 min, 1 hr, 3 hr, 5 hr, 8 hr | set the Automatic Power Off idle time | 33 |
| 17 | BUSY LED | On/Off | enable/disable the Busy LED | 33 |
| 18 | ARTS BEEP | In Range, Always, Off | select the beep option for ARTS operation | 40 |
| 19 | ARTS ITVL | 15 sec, 25 sec | set the polling interval for ARTS operation | 39 |
| 20 | KEY BEEP | On/Off | enable/disable the keypad beeper | 35 |
| 21 | EDGE BEEP | On/Off | enable/disable the band edge beeper | 51 |
| 22 | BELL | OFF, 1, 3, 5, 8, REPEAT | set the CTCSS bell ringer repetitions | 29 |
| 23 | MON/T-CAL | MON, T-CAL | select the MON key function | 29 |
| 24 | HOME/REV | HOME, REV | select the function of the HM/RV (EMG) key | 43, 65 |
| 25 | LAMP MODE | KEY, TOGGLE, 5 SEC | set the LCD/Keypad Lamp mode | 18 |
| 26 | TOT | OFF, 1 min, 2.5 min, 5 min, 10 min | set the TOT time (maximum length of a single transmission) | 31 |
| 27 | BCLO | On/Off | enable/disable the Busy Channel Lock-Out feature | 31 |
| 28 | CLOCK SFT | On/Off | Shifting of CPU clock frequency | 65 |
| 29 | SQL TYPE | OFF, TONE, TONE SQL, DCS | select the Tone Encoder and/or Decoder mode | 26 |
| 30 | TONE SET | 39 std CTCSS tones | set the CTCSS tone frequency, default is 100 Hz; note that you must press the 2 key to save settings, instead of PTT | 26 |
| 31 | DCS SET | 104 std DCS codes | set the DCS code, default is 023; note that you must press the 2 key to save settings, instead of PTT | 27 |
| 32 | DTMF SET |  | programming the DTMF autodialer | 36 |
| 33 | CW ID |  | program and activate the CW ID (Morse id) feature | 41 |
| 34 | SPEC-ANAL | SINGLE, CONTINUE | select the spectrum analyzer sweep mode | 53 |
| 35 | SMRT SRCH | SINGLE, CONTINUE | select the Smart Search sweep mode | 52 |
| 36 | RX MODE | AUTO, N-FM, W-FM, AM | select the operating mode | 23, 67 |
| 37 | LOCK MODE | KEY, DIAL, PTT, etc. | select the control locking combination to use | 34 |
| 38 | HALF DEV | On/Off | reduce the deviation level by 50% | 30 |
| 39 | LANGUAGE | English, Japanese | select the language for the Set (menu) mode | 39 |
| 40 | ON TIMER | Off to 24:00 | set the time until the unit will turn itself on | 34 |
| 41 | CONTRAST | 1 – 10 | set the display contrast level, default is 5 | 57 |
| 42 | MTR SYMB | >>, ■, ●,♦, ▬, = | select the S/PO meter symbol; default is >> | 56 |
| 43 | BARO OFST | -500 to +500 hpa | set the correction for barometric pressure; default is 0 hpa | 54 |
| 44 | ALTI OFST | -500 to +500 m | set the correction for altitude; default is 0 m | 54 |
| 45 | DISP MODE | NONE, TEMP, BARO, ALTV etc. | set the display for when the transceiver’s power is off | 57 |